## 👁️ SPHERE (Single-Vision) Silicone Hydrogel Contact Lenses

| Lens | Manufacturer | Material | Replacement Schedule | BC / DIA (mm) | Power Range (D) | Dk/t (×10⁻⁹ @ −3.00 D) | Notes / Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAILIES TOTAL1 | Alcon | Delefilcon A (Water-Gradient SiHy) | Daily disposable | 8.5 / 14.1 | +6.00 to −12.00 | 156 | MyAlcon — Premium comfort, highest O₂, silky surface. |
| Acuvue Oasys 1-Day | Johnson & Johnson Vision | Senofilcon A SiHy | Daily disposable | 8.5 or 9.0 / 14.3 | +8.00 to −12.00 | 121 | JNJ Vision Pro — HydraLuxe tech for digital comfort. |
| CooperVision MyDay Daily | CooperVision | Stenfilcon A (Aquaform SiHy) | Daily disposable | 8.4 / 14.2 | +8.00 to −12.00 | 100 | CooperVision — Soft modulus, high wettability. |
| Biofinity | CooperVision | Comfilcon A SiHy (48 % H₂O) | Monthly / 6 nights EW | 8.6 / 14.0 | +8.00 to −12.00 XR -20.00 to -12.50DS (0.50D steps) +8.50 to +15.00DS (0.50D steps) | 171 | CooperVision — Very high O₂; budget-friendly monthly. |
| Alcon TOTAL30 | Alcon | Lehfilcon A (Water-Gradient SiHy) | Monthly replacement | 8.4 / 14.2 | +8.00 to −12.00 | 154 | Alcon Science — Monthly version of TOTAL1, with “Celligent” anti-bacterial surface. |
| Acuvue Oasys 2-Week | Johnson & Johnson Vision | Senofilcon A SiHy | 2-week reusable / 6 nights EW | 8.4 or 8.8 / 14.0 | +8.00 to −12.00 | 147 | Optix-Now — Gold-standard fortnightly lens. |

## 🔄 TORIC Silicone Hydrogel Contact Lenses

| Lens | Manufacturer | Material | Replacement Schedule | BC / DIA (mm) | Sphere Range (D) | Cylinder / Axis Range | Dk/t (×10⁻⁹ @ −3.00 D) | Notes / Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAILIES TOTAL1 for Astigmatism | Alcon | Delefilcon A (Water-Gradient SiHy) | Daily disposable | 8.6 / 14.5 | +4.00 → −8.00 From -6.00 becomes 50 step | −0.75, −1.25, −1.75, −2.25 / 10°–180° 10degree steps | ≈127 | MyAlcon — Highest breathability daily toric; ultra-smooth surface; rotational stability. |
| Acuvue Oasys 1-Day for Astigmatism | Johnson & Johnson Vision | Senofilcon A (SiHy) | Daily disposable | 8.5 / 14.3 | +4.00 → −9.00 | −0.75 to −2.25 / 10°–180° | ≈129 | ACUVUE Specs — HydraLuxe tech; consistent orientation and comfort. |
| MyDay Toric | CooperVision | Stenfilcon A (Aquaform SiHy) | Daily disposable | 8.6 / 14.5 | +8.00 → −10.00 | −0.75 to −2.25 / 10°–180° | ≈80–100 | CooperVision UK — Soft, stable fit; good oxygen for daily wear. |
| Acuvue Oasys for Astigmatism (2-Week) | Johnson & Johnson Vision | Senofilcon A (SiHy) | 2-week reusable / ≤7 nights EW | 8.6 / 14.5 | +6.00 → −9.00 | −0.75 to −2.75 / 10°–180° | ≈129 | Optix-Now — Stable toric design; excellent balance of comfort & oxygen. |
| Biofinity Toric | CooperVision | Comfilcon A (48% H₂O SiHy)* | Monthly / ≤7 nights EW | 8.7 / 14.5 | +8.00 → −10.00 | −0.75 to −2.25 / 10°–180° 5 degree XR s 10 degrees | ≈116 | Optix-Now — Wide parameter range; stable ballast design; high Dk/t monthly option. |

## 👓 MULTIFOCAL (Presbyopia-Correcting) Silicone Hydrogel Lenses

| Lens | Manufacturer | Material | Replacement Schedule | BC / DIA (mm) | Power Range (D) | Add / Design Type | Dk/t (×10⁻⁹ @ −3.00 D) | Notes / Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAILIES TOTAL1 Multifocal | Alcon | Delefilcon A (Water-Gradient SiHy) | Daily disposable | 8.5 / 14.1 | +6.00 → −10.00 | LOW (+1.25), MED (+2.00), HIGH (+2.50) — Centre-near, precision-profile design | 156 | MyAlcon — Exceptional comfort, smooth power progression, excellent for mild → moderate presbyopes. |
| Acuvue Oasys 1-Day Multifocal | J&J Vision | Senofilcon A SiHy | Daily disposable | 8.4 / 14.3 | +6.00 → −9.00 | Adds: LOW (+0.75 – +1.25), MID (+1.50 – +1.75), HIGH (+2.00 – +2.50) — Pupil-optimised, centre-near aspheric design | 121 | JNJ Vision Pro — Designed for all pupil sizes; excellent near-to-distance vision. |
| Biofinity Multifocal | CooperVision | Comfilcon A SiHy (48 % H₂O) | Monthly / up to 7 nights EW | 8.6 / 14.0 | +6.00 → −8.00 | “Balanced Progressive™” Design — D (CN) & N (CD) lens pairs | 160–171 | CooperVision — Customisable near vs distance dominance; flexible monthly fit. |
| TOTAL30 Multifocal | Alcon | Lehfilcon A (Water-Gradient SiHy) | Monthly replacement | 8.4 / 14.2 | +8.00 → −10.00 | LOW, MED, HIGH adds — Precision-profile centre-near design | 154 | Alcon Science — Monthly upgrade of DAILIES TOTAL1 MTF; anti-bacterial “Celligent” surface for long-term comf |

Opti-Free PureMoist =

- Dual disinfectants (POLYQUAD + ALDOX) → broad-spectrum kill